01.03 
Maintenance program

Process Design & AMOS Configuration Document

Purpose

This Process Design document is provided to Key-Users to record their key decisions regarding the AMOS Process design and the required AMOS configuration.
It is updated throughout the Project Definition Phase and used as the baseline for the Final Acceptance prior to Go-Live

Revision

| Author, Name | Date | Notes |
| --- | --- | --- |
| Boris Kazarow | 01.06.2024 | First Version |
| Boris Kazarow | 18.12.2024 | Second Version: Ch. 1: Updated Maintenance Program Modes Ch. 6: Updated Flowchart Ch. 10: Preconditions updated |

Table of Contents

1.	Process Overview & Objectives	3

2.	Regulatory References	3

3.	Definitions & Glossary	4

4.	Process Tasks	6

5.	Process Dependencies	6

6.	Process Flow	7

7.	Process Design Questions	8

8.	Data (Source) Mapping	12

9.	Other Software Used	12

10.	AMOS Configuration	13

a.	Basic Data	13

b.	Parameters	13

c.	Workflows	15

d.	Scheduled Tasks	16

e.	API’s & Interfaces	17

11.	UAT Guidance	18

12.	Annex – Additional Info	18

# Process Overview & Objectives

Process General Description:

This process describes how Aircraft Maintenance Programs are created, revised, and archived in AMOS. A maintenance program consists of maintenance requirement items (tasks) of which plannable taskcards are created that produce and record all compliance relevant information.

Main Process Objective (Outputs & Outcomes):

The process objective is to translate aircraft, engine, and component Maintenance Requirements in accordance with MPD, competent Authority requirements as well as company policies into tasks resulting in an approved maintenance program.

Maintenance Programs can be created manually, via data transfer or OEM upload file (see Ch. 10 e).

Maintenance Program Taskcards can be grouped in pre-defined Checks (Defined in APN 281) for more convenient Planning and Execution purposes. Different Taskcard Types exist to control the planning and compliance behaviour.

Maintenance Programs can be managed in three different modes:

- Master Maintenance Program (MMP)

  - An MMP is applicable to an aircraft type that can be used for 2 or more Operator Maintenance Programs

  - Typically, the MMP represents the OEM MPD tasks which form the basis for the Operator Maintenance Program Taskcards, thus allowing a swifter Maintenance Program revision process.

  - Operator Maintenance Program Taskcards are fully customizable (time requirement, work template, requirement links) per operator and each OMP can be managed independently thus allowing operator specific authority approval processes.

  - MMP is created and updated in APN 249 and each OMP is updated and managed in APN 1934

- Operator Maintenance Program (OMP) / Single Operator Mode

  - An OMP is always assigned to one specific operator and therefore becomes individually manageable in APN 249 only

- Multi Operator Maintenance Program (MOMP)

  - This option can be used when 2 or more operators share the same approved maintenance program for an aircraft type (Example: European Airline Group’s fleet’s maintenance program being approved by EASA, ref. “One CAMO” regulation).

  - In this mode, aircraft transfers within such airline group becomes easier to manage as no maintenance program transfer and/or revision is required

  - MOMP is managed in APN 249 only

The activation of task, taskcard as well as Maintenance Program (MMP, OMP, MOMP) revisions can be controlled via the Approval Control framework (cf. Process 04.04 Approval Control).

The history of performed Taskcards can be retrieved from the following programs:

- Taskcard Status/History (APN: 673)

  - History of completions, Taskcard Interval Usage report.

- Taskcard Finding Report (APN: 876)

  - Findings per Taskcard, ratio, Avg. NRC Mhrs.

- Taskcard Times Report (APN: 876)

  - Avg. Mhrs per Taskcard.

# Regulatory References

EASA:

- Part M - Continuing Airworthiness Requirements

  - M.A.301 Continuing airworthiness tasks: This section outlines the responsibilities of the owner for the continuing airworthiness of the aircraft, including the need to ensure that a maintenance program is developed and adhered to.

  - M.A.302 Aircraft maintenance programme: This section details the requirements for an approved maintenance program. It states that the owner or operator must develop a maintenance program for the aircraft, which must be approved by the competent authority.

- Part CAMO - Continuing Airworthiness Management Organisation

  - CAMO.A.300 Continuing airworthiness management exposition: This section requires that the continuing airworthiness management organization (CAMO) include a description of the maintenance program in its exposition.

  - CAMO.A.315 Continuing airworthiness management: This section outlines the responsibilities of the CAMO in terms of managing the continuing airworthiness of the aircraft, including the development and approval of the maintenance program. FAA: Maintenance Programs for U.S.-Registered Aircraft Operated Under 14 CFR Part 129

FAA:

- 14 CFR Part 121 - Operating Requirements: Domestic, Flag, and Supplemental Operations

§ 121.367 Maintenance, preventive maintenance, and alterations programs: This section outlines the requirements for maintenance programs, including the necessity for operators to have an approved continuous airworthiness maintenance program (CAMP).

§ 121.373 Continuing analysis and surveillance: This section requires operators to have a system for the continuing analysis and surveillance of their maintenance programs.

- 14 CFR Part 135 - Operating Requirements: Commuter and On Demand Operations and Rules Governing Persons On Board Such Aircraft

§ 135.411 Applicability: This section indicates that each certificate holder must have a maintenance program.

§ 135.419 Approved aircraft inspection program: This section outlines the requirements for an approved aircraft inspection program.

# Definitions & Glossary

| Key-Words | Explanation |
| --- | --- |
| MRB | An MRB, or Maintenance Review Board, is a committee that develops and approves the Maintenance Review Board Report (MRBR), which is a document outlining the initial maintenance requirements for a new aircraft type. The MRB consists of representatives from the aviation regulatory authority (such as the FAA or EASA), the aircraft manufacturer, and operators and MROs. |
| MPD | The Maintenance Planning Document (MPD) contains all the MRB requirements together with the mandatory scheduled maintenance requirements. |
| MRI | Maintenance Requirement Items (or Tasks) are a list of Maintenance Requirements that originate from the MPD, Authority Requirements as well as Operator Specific Requirements. A Maintenance Requirement Item (Task) is not a plannable element; it must be linked to an actual Taskcard within the Maintenance Program. This program can be used to upload the MPD from the manufacturer and have it as reference in AMOS. |
| Maintenance Program | The Maintenance Program is the list of Taskcards. This list can contain MPD Taskcards; Company Taskcards, even N/A Taskcards. Of all Taskcard types, only Single Running Taskcards (= Out-of-Phase Tasks), Block Taskcards (= Phased Tasks) as well as On Demand Taskcards can be planned and executed. The other remaining taskcard types are event driven, i.e. they are not plannable hence not executable either, but receive the compliance information after the execution of a plannable event (Document, Part Requirement, Check, Taskcard) It is possible to create a new MP directly out of the MRI. With this process all the MP TC are linked to an MR item. |
| Single Running Taskcard | Single-Running Taskcards can be described as “Out-of-Phase” Taskcards. They are managed individually (i.e. not part of a Check) They are plannable events and therefore visible in the Maintenance Forecast (APN 1844). |
| Block Taskcard | Block Taskcards (BTC) are Taskcards that are grouped in a Check (i.e., via Check Control System (APN 281) and Check Package Builder (APN753). By default, they are not visible in the Maintenance Forecast (only the associated check appears there). However, in Maintenance Forecast (APN 1844) there is a possibility to activate the visibility of Block Taskcards in the Event Type filter panel. BTCs have their own next due. They can be individually deferred from a workpackage and also be planned individually out of the check they are linked to if required. |
| On Demand Taskcard | On Demand Taskcards (ODT) are frequently used in the event a maintenance event under a special condition (e.g. flight through volcanic ash or hard landing of aircraft) has to be performed. The ODT has an unlimited interval and can be planned via Maintenance Forecast (applying the correct filter in the time period panel) (APN 1844). |
| Task Controlled Task | Task Controlled Taskcards (TCT) are collecting the execution information from other Taskcards. |
| Rotable Controlled Taskcard | Rotable Controlled Taskcards (RTC) are used to track the performance of Time Controlled Component on the aircraft. Therefore, the Component needs a control limitation on the part number (part requirements) and will be linked thereafter to the RTC. |
| Document Controlled Taskcard | Document Controlled Taskcards (DCT) are controlled by a document in View/Edit Modifications (APN 63). The pending will be generated by the document itself and the compliance information following the execution of the document gets copied to the DCT. |
| Check Controlled Taskcard | Check Controlled Taskcards (CCT) are usually "easier" taskcards which can be performed within daily or weekly checks. The performance of the taskcard is done by a check maintenance event |
| Master Maintenance Program | In case of Multi-Operator Mode : A master maintenance program (MMP) is used when the MPs are managed in Multi-Operator Mode. This allows easier Maintenance Program revisioning by updating a root set of tasks that can update the same set of taskcards being part of multiple operator maintenance programs managed for different operators, under different authorities of a specific aircraft type. |
| Operator Maintenance Program | In case of Single-Operator Mode: The Taskcards defined in the Maintenance Program (APN: 249) are transferred Into OMP (APN 1934), where you can customize them as well as new Operator Taskcards can be created. Only the Taskcards with the status finished and the Worktemplate with the status active can be planned (once initialized). |
| Multi-Operator Maintenance Program | Case: Multi-Operator Mode sharing the same Maintenance Program : The management of this MOMP follows the same logic as in the Single-Operator Mode, however it is applicable to 2 or more than operators (AOC) allowing for easier aircraft transfers between AOCs. |
| Job Card | There two different types of Job Cards: - Preparation Job Card (special marking) - this job card is usually created to support another maintenance task and can be link to any maintenance event - Regular Job Card - this job card can be used to perform the actual work and can be signed individually from the maintenance event Jobcards are created and maintained in Work Template Manager (APN 1848), ref. Process 07.03 Worktemplates |
| MP Level (top level) | The main container of your maintenance program incl. the respective name and the affected aircraft type(s) |
| MP Issue Level | Issues are updated editions of the entire published Maintenance Program. An Issue contains all the Revisions which have been published since the previous Issue. |
| MP Revision level | A revision contains all taskcards of a Maintenance Program. There can only be one Revision with status "active". Older revisions of Maintenance Programs are archived. Revisions not activated - and therefore still editable - are called Tentative. If a Tentative Revision is activated the former revision is automatically archived. |

# Process Tasks

- Create/Update Maintenance Program (MMP / OMP / MOMP)

- Create/Update Tasks

- Create/Update Taskcards

- Create/Update Maintenance Checks

- Approve & Activate Maintenance Program

- Initialize Taskcards

# Process Dependencies

| INPUT | CONCURRENT | OUTPUT |
| --- | --- | --- |
| 01.07 Reliability Program | 02.01 Aircraft Phase-in 02.03 Multi operator A/C transfer 02.09 Airworthiness Certificate 02.07 Structural Damages 04.04 Approval Control 01.04 SB & AD Assessment 01.02 Part Definition | 02.06 Requirements Work Scoping 03.01 Planning 01.07 Reliability Program |

# Process Flow

# Process Design Questions

| # | Question | Answer |
| --- | --- | --- |
| 1 | How many MP do you manage? One per A/C Type/Fleet or per Customer/Operator? |  |
| 2 | Which MP(s) shall run in Single Operator Maintenance Program – Mode? Which MP(s) shall run in Multi Operator Maintenance Program – Mode (1 Master + X Operator Maintenance Programs (X>1)? Do you have multiple AOCs ? Do you use the EASA “One CAMO” possibility? |  |
| 3 | What is the approval process for a new MP revision? Note: On the Maintenance Program Revision you can print the Summary of Changes report. |  |
| 4 | How is your company currently approving the OMP/MMP externally? (explain the way how the paper is floating and who gives the signature?) |  |
| 5 | Which Taskcard numbering logic will be used? - Simple Number: any number can be entered (char max length = 20) - Composite Number: use a pre-defined Taskcard numbering logic. Note: If you are planning to use the import from an OEM upload file, it is recommended you should keep the same logic and numbering used by the OEM |  |
| 6 | How will you track component maintenance requirements? Note: It is recommended to set up the Part Requirement on the PN level (APN 2773) (cf. Process 01.02 Part Definition) and create an associated Rotable Controlled Taskcard linked to it |  |
| 7 | How will you track repetitive requirements coming from a document (e.g. AD/SB/STC/ICA) ? Note: It is recommended to set up the Document in APN 63 (cf. Process 01.04 SB/AD Assessment) and create an associated Document Controlled Taskcard linked to it Repetitive Requirements out of a Structural Damage (creating the SDR item as a document) can also be set up accordingly |  |
| 8 | How will your company revise a Maintenance Program? - Inside AMOS? - From an OEM file import? Is your OEM providing you a customized on your fleet or is the general one? |  |
| 9 | What is the data source of the Maintenance Program data that will be used for the revision process? (after Go-Live) Are Boeing (SGML), Boeing toolbox or Airbus (TSDF) files available? |  |
| 10 | Which Maintenance Program report layouts will be used? (Each Maintenance Program - Taskcard layout? Note: Some TC layouts can be customized in report designer (not all). Taskcard layouts are available for selection from the <Print Setup> tabsheet of each Maintenance Program. - Maintenance Program Summary? Note: Default Maintenance Program Summary report is configurable in the <Print Setup> tabsheet of each Maintenance Program. - Temporary Revision report? - Summary of Changes report? |  |
| 11 | Will the Maintenance Program Summary Pre-amble text, and Section Pre-amble text be entered in AMOS? |  |
| 12 | Do you check findings ratio to improve reliability of your MP? Note: Use Taskcard Finding Report APN 876 to review the ratio of Findings-W/O for each Taskcard. Moreover, you can: - Classify findings with a severity factor - Define CPCP tasks in <MSG codes> tabsheet. Findings on those tasks can be classified with Level1/2/3 finding - Set Finding possibilities |  |
| 13 | Do you review working time against tasks? Note: Use Taskcard Times Report (APN: 712) to review the Work-time used for each Taskcard execution. |  |
| 14 | Does your company track work to be carried out at the Step level on a Taskcard? |  |
| 15 | How will your company build checks? Sequential or independent? |  |
| 16 | Will your company use Ad-Hoc Checks (Check On Demand)? Note: Useful for example for engine changes, etc. You can set up the checks in the Check Control System (APN: 281) and then initialize the checks in the Maintenance Event Initializer (APN: 1895) Alternatively, Taskcards can be defined as on demand too. Use either Check or Taskcard as per your convenience and regulation. |  |
| 17 | Will you use AMOS Storage Engine to store the Taskcard as issued during a Check? Or will you use an external scan copy archive interfaced with AMOS? Note: See available interface with any scan archive system. |  |
| 18 | What Calculation Method will your company use and in which cases (which maintenance event types)? - Standard Calculation - Groundtime Extension - Flexible Calculation - Tolerance Extension Note: See the explanation on AMOS online help “Calculation Model” – Calculation of Due Date |  |
| 19 | Will the Go-Live migration done from the current legacy system or will it be a new MP from MPD import file? |  |

# Data (Source) Mapping

| Data Sources | Description | Volume / Manual / Automatic / Static / Dynamic |
| --- | --- | --- |

# Other Software Used

| Other Softwares | Used by: | Description |
| --- | --- | --- |

# AMOS Configuration

Preconditions

Within Address Administration (APN 53) all addresses used as Operator must have the property OC (Operator Code).

When revising Maintenance Program via file upload, some source data needs to be mapped to the following AMOS values, such as:

- Counter Dimension Codes (Defined in Counter Management APN 2031)

- Skill Definitions (=Plannable Resource Type, defined in APN 18 Basic Data Administration = Table "Planning Qualifications")

- Section Codes (=Maintenance Program Section, defined in APN 249 on MP header level)

- Type Codes (=Maintenance Task Code, defined in APN 249 on MP header level)

Following basic data for Maintenance Program/Taskcards to allow feeding the work template with information relevant to Production Planning :

  - Areas (APN 2627 Aircraft Code Definition)

  - Panels (APN 2627 Aircraft Code Definition)

  - Zones (APN 2627 Aircraft Code Definition)

  - Special Code (APN 249, MP header level)

This data can also be entered manually or via data transfer for each task and for each work template. This information is important for Production Planning purposes (cf. 07 Processes).

# Basic Data

| Basic Data Table | Notes |
| --- | --- |
| Aircraft Groups | Contains all A/C groups that can either be used to assign an aircraft to an A/C Group or define Taskcard Intervals per A/C group (e.g. Structural Sampling) |
| Check Package Sequence Code | Table with sequence codes that can be used to order taskcards within a check package (configurable in APN 2201) |
| Critical System Information | Contains all "Critical System Information" which can be used on a Taskcard item. |

# Parameters

| Parameter Code | Parameter Name | Notes |
| --- | --- | --- |
| 1165 | TASKCARD PRINT SETTINGS |  |
| 1189 | ALLOW COPY OF ALL TASKCARDS INTO ANOTHER MP REV. |  |
| 1217 | ENABLE TRACKING OF TASKCARD CHANGES |  |
| 1243 | CREATE NEW TC REVISION FOR TEMP. ACT. TCS |  |
| 1284 | CHANGE TASKCARD NUMBER FUNCTIONALITY |  |
| 1323 | ALLOW MULTIPLE TASKCARD PRINTING |  |
| 1337 | WATERMARK FOR PREVIEW TASKCARDS |  |
| 1339 | SAMPLE BEHAVIOUR OF SINGLE RUNNING TASKCARDS |  |
| 1402 | USE OF TASKCARD NOT APPLICABLE FLAG |  |
| 1428 | TASKCARD MODIFICATION STATUS EFFECTIVITY |  |
| 1441 | RESCHEDULE SRTS OF NON-MANAGED A/C |  |
| 1483 | AMM REFERENCE NAME |  |
| 1530 | CHANGE TC TO REPORT DESIGNER LAYOUT |  |
| 1531 | OEM TASKCARD REFERENCE NAME |  |
| 1541 | OEM TC SET PRINT ON PRINT TO ALLOWED(DEFAULT: PRINT) |  |
| 1571 | SHOW MANHOURS(MRB, REQUIRED SKILL FOR RELEASER, REQUIRED SKILL FOR DBL. RELEASER |  |
| 1573 | SHOW RELEASE SKILLS FOR TASKCARD |  |
| 1580 | SHOW RELATED MRIS |  |
| 1660 | S1000D REFERENCE NAME |  |
| 1646 | TRANSFER MMP REVISION INFORMATION TO OMP FROM APN1934 |  |
| 1657 | TASKCARD LOADING BEHAVIOUR |  |
| 1663 | TASKCARD WORKTEMPLATE EDITING IN MAINTENANCE PROGRAM ADMINISTRATION |  |
| 1701 | HANDLE TASKCARDS CREATION FOR N/A TASKS |  |
| 1719 | ENABLE/DISABLE APPROVAL CONTROL IN MAINTENANCE PROGRAM ADMINISTRATION |  |
| 1720 | ENABLE/DISABLE APPROVAL CONTROL IN OPERATOR MAINTENANCE PROGRAM |  |
| 1743 | ENABLE AUTO-REPORT BACK (ID:1743) |  |

# Workflows

| Workflow Number | Workflow Name | Notes |
| --- | --- | --- |
| 29 | New Maint. Program Revision activated | to distribute information regarding new MP amongst pre-defined group |
| 30 | Temporary Inactive Taskcard/MRI's activated | This Workflow can be used to inform users when an inactive Taskcard Revision has been activated in APN 249 |
| 61 | Changes done on an MRI | This Workflow can be used to inform users when a MRI has been changed, newly created or removed in APN 249 |
| 69 | Changes done on a Taskcard | This Workflow can be used to inform users when a Taskcard has been changed, newly created or removed in APN 249 |
| 80 | Taskcard Document Link | This workflow is triggered if the taskcard data was changed and the taskcard is linked to a document. |
| 384 | Task save | This workflow is triggered and checked when a Task is selected. It can be used to control the editing of the Task by using the Approval State wildcards and the “Conditional Abort” action. |
| 385 | Taskcard save | This workflow is triggered and checked when a Taskcard is selected. It can be used to control the editing of the Taskcard by using the Approval State wildcards and the “Conditional Abort” action. |
| 230 | Worktemplate Modification | The workflow will be triggered whenever a Work Template is modified. The following actions are possible: CHANGED, DELETED or NEW (for each action a Template Rule exists). To differentiate between the Work Template Requirement References the wildcard `event_type` can be used. |

# Scheduled Tasks

| Scheduled Task Reference | Frequency | Notes |
| --- | --- | --- |
| ExtractTaskcardDueValues | Only on demand | Export function |
| MRIsWithNoTaskcardRelation | N/A | This scheduler Task is only required if an MRI Maintenance Program is used. |
| MaintenanceRequirementInitControl | Optional. It is recommended to run this scheduler task once a week but it can also be configured with a lower frequency. |  |
| TaskcardForecastService | Once a day | This scheduled task is recommended and strongly encouraged for performance and stability reasons This scheduled task calculates the ToGo, Since Req and Expected Date of all events related to an Aircraft. Data is filled in the forecast table. |
| TimeRequirementConsistencyService | It is recommended to run this task at least every two days. | This scheduler task is used to check the data consistency for Time Requirement definition which are linked to different Maintenance Requirements. |
| BOHistoryUpdateTask | Proposed: once per month | This service creates missing history entries and re-link existing history entries to the correct level. |
| EffectivityApplicabilityService | Proposed: each night | To check and resolve the Effectivity Applicability Table |
| EffectivityConsistencyTask | Recommended: once a week | Identify all Consistency issues for Effectivity definitions for all Requirements. |
| RequirementLinkCodePrintModeCheckTask | On demand |  |

# API’s & Interfaces

| Reference | Direction | Trigger | Frequency |
| --- | --- | --- | --- |
| Boeing SGML | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| Boeing MPD 05 | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| Boeing Toolbox (Schema v.2.5.5) | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| Airbus TSDF Rev. 07 | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| Airbus SGML | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| LHT Maintenance Schedule Import | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| OEM Document Import (Beta) | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| AMOS MP Revision | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |
| Airbus Helicopters ALS / MSM | Import | Manual trigger from APN 249 | On demand (at every new MPD revision embodiment) |

# UAT Guidance

| No | Proposed User Acceptance Test scenario |
| --- | --- |
| 1 | Create new operator maintenance program for new aircraft type (All TC types) |
| 2 | Create two operator maintenance programs based on the same A/C type master MP. |
| 3 | Complete new revision of maintenance program (via import file) |
| 4 | Revise TC (intervals or effectivity) + initialize |
| 5 | Revise TC worktemplate |
| 6 | Deactivation of TC during revision process |
| 7 | Define checks + assign TC to checks + activate checks Deassign TC from checks and update pending events |

# Annex – Additional Info